# Software Testing and Reliability

A Short Course @ Guidant Corporation, Minneapolis/St Paul

Aug 12-16, 2002

In Class Exercise# 02   Date assigned: 08/12/02

Prepared by: Ramkumar Natarajan

- Objective:

Draw state machines for a specification.

- Problem Statement:

We will do a couple of small exercises to familiarize the concepts behind finite state machines.

- Draw a state machine that accepts any string of 0s and 1s (zeroes and ones), with an even number of zeroes and ones.  (Hint:  Create a state machine with transitions based on the next character in the string and make your start and final states such that the requirements are met).

- Given below is a description of a seat belt alarm system in an automobile.  Draw it’s corresponding FSM.

If the driver:

  - Turns on the key and

  - Does not fasten the seat belt within 5 seconds

Then sound the alarm

- For 5 seconds or

- Until the driver fastens the seat belt or

- Until the driver turns off the key.